# Custom PID Guide - OBD2App

## Overview

Custom PIDs allow you to read extended OBD-II parameters that aren’t included in the standard Mode 01 PIDs. This includes manufacturer-specific diagnostics, enhanced parameters, and data from non-engine ECUs.

## Table of Contents

- What Are Custom PIDs?

- Getting Started

- Configuring Custom PIDs

- Understanding Formulas

- Real-World Examples

- Discovering Custom PIDs with Terminal Apps

- Troubleshooting

- Advanced Tips

## What Are Custom PIDs?

### Standard vs Custom PIDs

| Feature | Standard PIDs | Custom PIDs |
| --- | --- | --- |
| Discovery | Automatic (bitmask queries) | Manual configuration |
| Mode | Mode 01 only | Any mode (21, 22, 23, etc.) |
| Headers | Default (7DF) | Any ECU header (7E0, 7E1, 760, etc.) |
| Availability | Universal (all vehicles) | Manufacturer/vehicle specific |
| Examples | RPM, speed, coolant temp | Yaw rate, hybrid battery SOC, transmission temp |

### Use Cases

- Manufacturer-specific diagnostics (Jaguar, BMW, Toyota)

- Enhanced parameters (Mode 22 extended data)

- Non-engine ECUs (transmission, ABS, instrument cluster)

- Hybrid/EV data (battery state, inverter temps)

- Performance monitoring (boost pressure, G-forces)

## Getting Started

### Step 1: Create a Vehicle Profile

Custom PIDs are stored per vehicle profile:

- Open OBD2App → Settings tab

- Tap + Add to create a new profile or edit existing one

- Fill in vehicle details:

  - Profile name (e.g., “Jaguar XF 2020”)

  - Fuel type, engine size, tank capacity

  - Engine displacement and volumetric efficiency (for fuel calculations)

- Save the profile

### Step 2: Access Custom PID Management

- In Settings, tap to edit your vehicle profile

- Tap “Manage Custom PIDs” button (appears for existing profiles only)

- You’ll see the Custom PID list (empty initially)

## Configuring Custom PIDs

### Adding a New Custom PID

Tap + Add to create a new custom PID:

| Field | Description | Example | Notes |
| --- | --- | --- | --- |
| Name | Human-readable display name | “Yaw Rate” | Include units if helpful |
| Header | ECU header in hex | “760” | Leave blank for default (7DF) |
| Mode | OBD mode in hex | “22” | Default is “22” for extended |
| PID | Parameter ID in hex | “0456” | Must include leading zeros |
| Bytes | Number of data bytes | 2 | Usually 1-8 bytes |
| Unit | Display unit (optional) | “°/s” | Appears in UI |
| Formula | Arithmetic expression | ((A*256)+B)/100 | See formula section |
| Signed | Interpret bytes as signed | unchecked | Important for negative values |

### Managing Existing PIDs

- Tap any PID in the list to edit it

- Use delete button in edit mode to remove

- PIDs are automatically saved when you tap Save

## Understanding Formulas

The formula converts raw OBD data bytes to meaningful values.

### Variables

- A, B, C, D, E, F, G, H = Data bytes 1 through 8

- Each variable contains the decimal value (0-255) of the corresponding byte

### Operations

- Basic: +, -, *, /

- Parentheses: () for grouping

- Order of operations: Standard mathematical precedence

### Common Formula Patterns

| Pattern | Description | Example |
| --- | --- | --- |
| A-40 | Standard temperature conversion | Coolant temp: A-40 |
| ((A*256)+B)/100 | 16-bit value divided by 100 | Yaw rate: ((A*256)+B)/100 |
| A*0.078125-40 | Toyota-specific temp conversion | Sensor temp: A*0.078125-40 |
| A*0.5 | Simple scaling | Battery SOC: A*0.5 |
| ((A*256)+B)-32768 | 16-bit signed with offset | Steering angle: ((A*256)+B)-32768 |

### Formula Examples

// Engine coolant temperature (standard)
"A-40"

// 16-bit RPM calculation  
"((A*256)+B)/4"

// Percentage with decimal precision
"A*0.5"

// Signed angle with center offset
"((A*256)+B)-32768"

// Complex scaling
"((A*256)+B)*0.015625"

## Real-World Examples

### Example 1: Jaguar XF Yaw Rate

| Field | Value |
| --- | --- |
| Name | “Yaw Rate” |
| Header | “760” |
| Mode | “22” |
| PID | “0456” |
| Bytes | 2 |
| Unit | “°/s” |
| Formula | ((A*256)+B)/100 |
| Signed | unchecked |

Terminal Response: 62 04 56 01 F4
Calculation: ((1×256)+244)/100 = 5.0 °/s

### Example 2: Toyota Hybrid Battery SOC

| Field | Value |
| --- | --- |
| Name | “Hybrid Battery SOC” |
| Header | “7E4” |
| Mode | “22” |
| PID | “01DB” |
| Bytes | 1 |
| Unit | “%” |
| Formula | A*0.5 |
| Signed | unchecked |

Terminal Response: 62 01 DB B4
Calculation: 180 × 0.5 = 90.0%

### Example 3: BMW Transmission Temperature

| Field | Value |
| --- | --- |
| Name | “Transmission Temp” |
| Header | “7E1” |
| Mode | “22” |
| PID | “10B8” |
| Bytes | 1 |
| Unit | “°C” |
| Formula | A-40 |
| Signed | unchecked |

Terminal Response: 62 10 B8 58
Calculation: 88 - 40 = 48°C

## Discovering Custom PIDs with Terminal Apps

### Recommended Android Terminal Apps

- Serial Bluetooth Terminal (by Kai Morich) - Free, popular

- Bluetooth Terminal HC-05 - Simple interface

- Serial Monitor - Good for logging

- Termius - Advanced features

### Step-by-Step Discovery Process

#### 1. Connect to Your OBD-II Adapter

1. Pair adapter in Android Bluetooth settings
2. Open terminal app
3. Connect to your OBD-II adapter

#### 2. Initialize the Adapter

ATZ         # Reset adapter (responds with version)
AT E0       # Echo off
AT L0       # Linefeeds off  
AT S0       # Spaces off
AT SP 0     # Set protocol to auto
AT DP       # Show current protocol

#### 3. Verify Standard Communication

010C        # Engine RPM (should respond: 41 0C XX XX)
010D        # Vehicle speed (should respond: 41 0D XX)

#### 4. Explore Different ECU Headers

AT SH 7E0  # Engine ECU (default)
AT SH 7E1  # Transmission ECU
AT SH 7E2  # ABS ECU  
AT SH 760  # Instrument Cluster (Jaguar/LR)
AT SH 7E4  # Hybrid ECU (Toyota/Lexus)
AT SH 7DF  # Default broadcast header

#### 5. Try Extended Mode 22 PIDs

# Systematic approach
2200        # Extended PID 00
2201        # Extended PID 01
2202        # Extended PID 02
# Continue through 22FF

#### 6. Try Other Modes

# Mode 21 - Transmission data
2100        # Transmission PID 00
2101        # Transmission PID 01

# Mode 23 - Manufacturer specific
2300        # Manufacturer PID 00

### Understanding Terminal Responses

#### Standard PID Response

Command: 010C (Engine RPM)
Response: 41 0C 0A F0
         ^^ ^^ ^^^^^
         || || |||||
         || || ||+++-- Data bytes (0A F0 = 2800 RPM)
         || || |+----- Byte count
         || |+------- PID (0C)
         |+--------- Mode (41 = 01 + 0x40)
         +----------- Response prefix

#### Extended PID Response

Command: 220456 (Yaw rate)
Response: 62 04 56 01 F4
         ^^ ^^ ^^ ^^^^^
         || || || |||||
         || || || ||+++-- Data bytes (01 F4)
         || || || |+----- Byte count
         || || |+------- PID low byte (56)
         || |+--------- PID high byte (04)  
         |+----------- Mode (62 = 22 + 0x40)
         +------------- Response prefix

### Common Response Meanings

| Response | Meaning |
| --- | --- |
| 41 XX ... | Valid Mode 01 response |
| 62 XX ... | Valid Mode 22 response |
| NO DATA | PID not supported by this ECU |
| UNABLE TO CONNECT | Wrong ECU header |
| ERROR or ? | Communication error |

### Systematic Discovery Strategy

- Research First

  - Vehicle-specific forums

  - Service manuals

  - Torque app PID database

- Start with Known Headers

- # Common headers to try
AT SH 7E0  # Most vehicles
AT SH 7E1  # Transmission
AT SH 760  # Jaguar/Land Rover
AT SH 7E4  # Toyota/Lexus hybrids

- Document Everything

- | Header | Mode | PID | Response | Description | Formula |
|--------|------|-----|----------|-------------|---------|
| 760    | 22   | 0456| 62 04 56 01 F4 | Yaw rate | ((A*256)+B)/100 |

- Test While Driving

  - Some PIDs only work when vehicle is moving

  - Transmission data needs gear engagement

  - ABS data needs wheel movement

## Troubleshooting

### Custom PID Shows “NODATA”

| Cause | Solution |
| --- | --- |
| Wrong ECU header | Try different headers (7E0, 7E1, 760, etc.) |
| Incorrect mode/PID | Verify hex values with terminal app |
| Vehicle not ready | Start engine, wait for initialization |
| ECU not present | Vehicle may not have this ECU |

### Values Look Wrong

| Issue | Fix |
| --- | --- |
| Incorrect formula | Test calculation manually |
| Wrong byte count | Adjust bytes returned |
| Signed/unsigned mismatch | Toggle signed checkbox |
| Endianness wrong | Try (B*256)+A instead of (A*256)+B |

### Terminal App Issues

| Problem | Solution |
| --- | --- |
| Can’t connect | Check Bluetooth pairing, try different terminal app |
| No responses | Verify adapter initialization (ATZ, AT E0) |
| Garbled text | Check baud rate, try AT SP 0 for auto protocol |

## Advanced Tips

### Performance Optimization

- Group PIDs by Header - Minimizes AT SH switches

- Use Slow Tier - Custom PIDs polled every 5th cycle

- Limit Byte Count - Only request needed bytes

- Test Formulas - Use unit test framework

### Documentation

- Save PID Sources - Note where you found each PID

- Vehicle Notes - Document year/model/ECU variations

- Formula Derivation - Explain complex calculations

- Test Conditions - Note when PIDs are valid

### Sharing Configurations

- Export Profiles - Share vehicle profiles with custom PIDs

- Community Contributions - Share findings on forums

- Version Control - Track changes to PID configurations

- Backup Regularly - Prevent loss of custom configurations

### Experimental Approaches

# Brute force script (if terminal supports scripting)
for pid in 2200..22FF; do
    echo "Testing PID $pid"
    send "$pid"
    sleep 0.1
done

# Header switching test
headers=("7E0" "7E1" "7E2" "760" "7E4")
for header in "${headers[@]}"; do
    echo "Testing header $header"
    send "AT SH $header"
    send "2200"
done

## Resources

### PID Databases

- Torque App PID Database

- OBD-II PIDs Wikipedia

- Vehicle-specific forums (Jaguar, BMW, Toyota, etc.)

### Documentation

- Vehicle service manuals

- Manufacturer technical bulletins

- SAE J1979 standard (OBD-II specification)

### Tools

- Android terminal apps (Serial Bluetooth Terminal)

- PC tools (ScanTool, OBDwiz)

- Professional diagnostic software (manufacturer specific)

## Conclusion

Custom PIDs unlock a wealth of vehicle data beyond standard OBD-II parameters. While they require research and manual configuration, they provide access to manufacturer-specific diagnostics, enhanced parameters, and data from non-engine ECUs.

The combination of OBD2App’s custom PID system and terminal-based discovery gives you powerful tools to explore your vehicle’s data capabilities. Start with well-documented PIDs for your vehicle, then expand your configuration as you discover new parameters.

Happy hacking! 🚗💨