## Custom PID Functionality - User Guide

What Are Custom PIDs?

Custom PIDs allow you to read extended OBD-II parameters that aren't included in the standard Mode 01 PIDs. This includes:

- Manufacturer-specific PIDs (like Jaguar, BMW, Toyota)

- Enhanced diagnostics (Mode 22, Mode 21, etc.)

- Special ECUs with custom headers (e.g., transmission, ABS)

Step 1: Create/Edit a Vehicle Profile

Custom PIDs are stored per vehicle profile. First, ensure you have a vehicle profile:

- Go to Settings tab

- Tap + Add to create a new profile or edit an existing one

- Fill in vehicle details (name, fuel type, engine size, etc.)

- Save the profile

Step 2: Access Custom PID Management

- In Settings, tap to edit an existing vehicle profile

- You'll see a "Manage Custom PIDs" button (only appears for existing profiles)

- Tap this button to open the Custom PID list

Step 3: Add a Custom PID

Tap + Add to create a new custom PID:

| Field | Description | Example |
| --- | --- | --- |
| Name | Human-readable name for display | "Yaw Rate" |
| Header | ECU header (hex). Leave blank for default (7DF) | "760" |
| Mode | OBD mode (hex). Default is "22" for extended | "22" |
| PID | Parameter ID (hex) | "0456" |
| Bytes | Number of data bytes returned | 2 |
| Unit | Unit for display (optional) | "°/s" |
| Formula | Arithmetic expression to convert raw bytes | ((A*256)+B)/100 |
| Signed | Whether to interpret bytes as signed | Unchecked |

Step 4: Understanding Formulas

The formula converts raw OBD data bytes to meaningful values:

- Variables: A, B, C, D, E, F, G, H (represent bytes 1-8)

- Operations: +, -, *, /, parentheses ()

- Examples:

  - A-40 (standard coolant temp conversion)

  - ((A*256)+B)/100 (16-bit value divided by 100)

  - (A*0.078125)-40 (Toyota-specific temp conversion)

Step 5: Real-World Example - Jaguar XF

Here's how to add Jaguar XF's yaw rate sensor:

- Name: Yaw Rate

- Header: 760 (instrument cluster ECU)

- Mode: 22 (extended diagnostics)

- PID: 0456

- Bytes: 2

- Unit: °/s

- Formula: ((A*256)+B)/100

- Signed: unchecked

This will read the yaw rate in degrees per second.

Step 6: Using Custom PIDs

Once configured:

- Connect to your vehicle via Bluetooth

- Custom PIDs are automatically polled in the slow tier (every 5th cycle)

- Results appear in the Details tab alongside standard PIDs

- You can add these to dashboard widgets just like standard PIDs

Step 7: Managing Custom PIDs

In the Custom PID list:

- Tap any PID to edit it

- Use the delete button in edit mode to remove

- Toggle enabled/disabled (planned feature)

Finding Custom PIDs for Your Vehicle

Sources:

- Vehicle-specific forums (e.g., Jaguar, BMW, Toyota forums)

- OBD-II documentation for your make/model

- Torque app PID database (many user-contributed PIDs)

- Service manuals with diagnostic procedures

- Wireshark captures from manufacturer tools

Common Manufacturer PIDs:

- Toyota/Lexus: Mode 22 PIDs for hybrid battery, inverter temps

- Jaguar/Land Rover: Body control module, instrument cluster PIDs

- BMW: Transmission, DME-specific PIDs

- VW/Audi: ABS, ESP module PIDs

Step 8: Troubleshooting

If Custom PID Shows "NODATA":

- Check header - wrong ECU header

- Verify mode/PID - incorrect values

- Test with AT commands in terminal app

- Check vehicle compatibility - may not be supported

If Values Look Wrong:

- Review formula - incorrect math

- Check byte count - too few/many bytes

- Verify signed/unsigned - interpretation issue

- Test with known values - compare with manufacturer tools

Advanced Tips

- Group by Header: Put PIDs with same header together for efficiency

- Use Descriptive Names: Include units in name for clarity

- Document Sources: Note where you found each PID

- Test Incrementally: Add one PID at a time to verify

- Backup Profiles: Export profiles to share between devices

Example: Toyota Hybrid Battery SOC

For a Toyota Prius hybrid battery State of Charge:

- Name: Hybrid Battery SOC

- Header: 7E4 (hybrid ECU)

- Mode: 22

- PID: 01DB

- Bytes: 1

- Unit: %

- Formula: A*0.5

- Signed: unchecked

This gives you the battery percentage in real-time!

The custom PID system makes your OBD2 app incredibly flexible - you can add virtually any parameter your vehicle's ECUs expose, not just the standard ones.

Note: the system will NOT automatically discover custom PIDs.

Custom/Extended PIDs - Manual Configuration Only

Custom PIDs (Mode 22, custom headers, etc.) cannot be auto-discovered because:

- No Standard Discovery Mechanism: Unlike Mode 01, extended modes don't have discovery queries

- Manufacturer-Specific: Each manufacturer defines their own PIDs

- ECU-Dependent: Different ECUs (engine, transmission, ABS) have different PIDs

- Header Variations: Need to know the correct ECU header (7E0, 7E1, 760, etc.)

What Gets Discovered Automatically

| PID Type | Discovery Method | Example |
| --- | --- | --- |
| Standard Mode 01 | Automatic bitmask queries | RPM (010C), Speed (010D), Temp (0105) |
| Custom Mode 22 | Manual only | Jaguar yaw rate (220456) |
| Custom Headers | Manual only | Toyota hybrid ECU (7E4) |
| Mode 21/23/etc | Manual only | Manufacturer diagnostics |

Practical Example

When you connect to a Jaguar XF:

What You'll See Automatically:

- Engine RPM (010C)

- Vehicle speed (010D)

- Coolant temp (0105)

- Throttle position (0111)

- All other standard Mode 01 PIDs the ECU supports

What You WON'T See Unless Configured:

- Yaw rate (220456 from header 760)

- Lateral acceleration (220457 from header 760)

- Steering angle (220458 from header 760)

- Transmission temp (Mode 22, header 7E1)

How to Find Your Vehicle's Custom PIDs

1. Research Your Vehicle

- Forums: Jaguar, BMW, Toyota enthusiast forums

- Documentation: Service manuals, technical bulletins

- Tools: Manufacturer diagnostic software

- Community: Torque app PID database, OBD-II forums

2. Experimental Approach

bash

# Using a terminal app, try common extended PIDs:

AT SH 760    # Switch to instrument cluster

AT SH 7E4    # Switch to hybrid ECU

2200         # Query PID 00 in Mode 22

2201         # Query PID 01 in Mode 22

# etc...

3. Reverse Engineering

- Capture traffic from manufacturer tools

- Use Wireshark to analyze CAN bus messages

- Study open-source tuning software databases

Summary

- Standard PIDs: ✅ Auto-discovered (no work needed)

- Custom PIDs: ❌ Must be manually configured per vehicle

- Discovery Limitation: This is an OBD-II protocol limitation, not app limitation

- Benefit: Manual configuration gives you access to MUCH more data than standard PIDs

The custom PID system is powerful precisely because it lets you access data that automatic discovery can't reach - but it requires research and manual configuration for each vehicle model.