# OBD2 Viewer — Quick Start User Guide

Your Complete Vehicle Diagnostics & Trip Computer

## 📱 What is OBD2 Viewer?

OBD2 Viewer transforms your Android phone into a powerful vehicle diagnostics tool and trip computer. Connect to your car’s OBD-II port via Bluetooth, view live engine data, track trips with GPS, and analyze your driving with customizable dashboards.

## 🚀 Quick Start (5 Steps)

### 1. Hardware Setup

- Plug your ELM327 Bluetooth adapter into your vehicle’s OBD-II port (usually under the dashboard near the steering wheel)

- Turn on your vehicle’s ignition (engine can be off for initial setup)

- Pair the adapter with your Android device via Settings → Bluetooth (typically shows as “OBDII” or “ELM327”)

### 2. First Launch

- Open OBD2 Viewer

- Grant Bluetooth and Location permissions when prompted

- Enable Bluetooth if prompted

### 3. Connect to Your Vehicle

- Tap the Connect tab

- Find your paired OBD-II adapter in the device list

- Tap the device name to connect

- Wait for “Connected” status (takes 5-10 seconds)

### 4. Create Your Vehicle Profile (Optional but Recommended)

- Go to Settings tab

- Tap “+ Add Profile”

- Enter: Vehicle name, fuel type, tank capacity, fuel price

- Advanced: Engine power (BHP), vehicle mass (kg) for power calculations

- Tap Save

### 5. Start Your First Trip

- Go to Trip tab

- Tap “Start Trip” button

- Drive normally — the app tracks everything automatically

- Tap “Stop Trip” when done

## 📊 Main Features

### 🎯 Trip Computer

- Real-time metrics: Distance, duration, fuel used, average speed

- Fuel efficiency: Instant and trip average (L/100km or km/L)

- Drive mode analysis: % time in city/highway/idle

- GPS tracking: Accurate distance and speed with satellite positioning

- Trip logging: Save detailed JSON logs for analysis

### 📈 Live Dashboard

- Customizable layouts: Create unlimited dashboard configurations

- 6 widget types: Dial gauges, 7-segment displays, bar gauges, numeric displays, temperature arcs

- 50+ metrics: RPM, speed, temperatures, fuel, power, acceleration, and more

- Color schemes: Dark, Neon Red, Green LCD themes

- Warning zones: Set thresholds for visual alerts

### 🔧 Diagnostics

- OBD-II parameters: Engine load, coolant temp, throttle, MAF, fuel trim, O2 sensors, and more

- Auto-discovery: Only displays parameters your vehicle supports

- Details view: Complete table of all live sensor readings

- Custom PID support: Add manufacturer-specific and extended OBD-II parameters

- PID discovery: Built-in discovery tool for finding supported custom PIDs

- Multiple ECU support: Access data from engine, transmission, ABS, and other ECUs

### 📍 GPS Integration

- Accurate tracking: Uses Google Play Services location

- Altitude correction: MSL altitude with geoid correction

- Satellite count: Shows GPS signal quality

- Speed validation: Cross-checks OBD speed vs GPS speed

### �️ Trip Map View

- Visual trip tracking: View recorded trips on OpenStreetMap

- Complete route: Shows full trip path with start and end markers

- Interactive scrub bar: Move cursor along the trip timeline

- Sample details: Tap “Details” to view full JSON data at any point

- GPSLogger-style cursor: Black marker with white ring for visibility

- Color-coded markers: Green start, red end markers with white borders

### � Accelerometer Analysis (Optional)

- Road quality tracking: Detects bumps and potholes

- G-force measurement: Forward/lateral/vertical acceleration

- Lean angle: Vehicle tilt during cornering

- Brake/acceleration peaks: Maximum forces recorded

## 🎮 Using the App

### Navigation

Swipe between 5 main screens or use the bottom navigation bar: 1. Trip — Start/stop trips, view live stats 2. Connect — Bluetooth device connection 3. Dashboards — Customizable gauge layouts 4. Details — Full OBD-II parameter table 5. Settings — Vehicle profiles and preferences

Map View is accessible from the Trip Summary screen after recording a trip.

### Recording a Trip

Start: 1. Ensure OBD-II adapter is connected (green indicator on Trip screen) 2. Tap “Start Trip” 3. Drive normally — all data is recorded automatically 4. App runs in background with persistent notification

Pause/Resume: - Tap “Pause” to temporarily stop recording (e.g., at a gas station) - Tap “Resume” to continue the same trip - Paused time is excluded from trip calculations

Stop: - Tap “Stop Trip” to end recording - Trip log is automatically saved (if logging enabled in Settings) - View trip summary before closing

### Viewing Trip on Map

Open Map View: 1. After stopping a trip, view the Trip Summary screen 2. Tap the map icon in the top bar 3. Map loads showing your complete trip route

Using Map View: - Green marker: Trip start point - Red marker: Trip end point
- Black cursor: Current position on timeline - Scrub bar: Drag to move along trip timeline - Details button: View full JSON data at cursor position - Back navigation: Use Android back button to return to Trip Summary

Sample Details: - Tap “Details” to see complete sensor data at any point - Scroll through JSON data with vertical scrollbar - Copy JSON text to clipboard using “Copy JSON” button

### Creating Custom Dashboards

- Go to Dashboards tab → Tap menu (⋮) → “New Layout”

- Choose a name and color scheme

- Tap “+ Add Widget”

- Select:

  - Metric (e.g., RPM, Speed, Coolant Temp)

  - Widget type (Dial, 7-Segment, Bar, etc.)

  - Range (min/max values, auto-populated)

  - Warning threshold (optional)

- Drag widget to position, resize using corner handles

- Tap Save when done

- Switch between layouts via the layout list

### Managing Vehicle Profiles

Why use profiles? - Different vehicles have different fuel types, tank sizes, and engine specs - Profiles enable accurate fuel cost and power calculations - Set custom OBD polling rates per vehicle - Store custom PID configurations per vehicle

Create a profile: 1. Settings → “+ Add Profile” 2. Fill in basic info (name, fuel type, tank capacity, fuel price) 3. Optional: Engine power (BHP) and vehicle mass (kg) for power metrics 4. Tap Save

Switch profiles: - Tap any profile in the list to make it active - Active profile shows a blue badge

Custom PID Management: 1. Edit an existing vehicle profile 2. Tap “Manage Custom PIDs” button 3. Add manufacturer-specific PIDs, extended parameters, and non-engine ECU data 4. Configure formulas to convert raw data to meaningful values 5. Use built-in discovery tool to find supported PIDs

## ⚙️ Settings Explained

| Setting | What it does |
| --- | --- |
| OBD Connection | Toggle between real OBD adapter and mock mode (for testing) |
| Auto-connect | Automatically connect to last used device when opening Connect screen |
| Enable Logging | Save trip data to JSON files in selected folder |
| Auto-share Log | Automatically open share dialog after trip ends |
| Log Folder | Choose where trip logs are saved (default: Downloads) |
| Accelerometer | Enable road quality and G-force tracking (uses more battery) |
| Bluetooth Logging | Save raw Bluetooth communication for debugging |
| Custom PID Discovery | Scan for supported manufacturer-specific PIDs |

## 💡 Tips & Best Practices

### For Best Results:

✅ Keep GPS enabled — Required for accurate distance and speed
✅ Create a vehicle profile — Enables fuel cost and power calculations
✅ Start trip after engine starts — Ensures all sensors are active
✅ Keep phone charged — GPS + Bluetooth drain battery faster
✅ Use a phone mount — Safely view dashboard while driving
✅ Explore custom PIDs — Unlock manufacturer-specific data for your vehicle

### Troubleshooting:

Can’t connect to adapter? - Verify adapter is paired in Android Bluetooth settings first - Ensure vehicle ignition is ON - Try unplugging and re-plugging the adapter - Check adapter LED is blinking (indicates power)

No GPS signal? - Ensure Location permission is granted - Move away from buildings/tunnels for initial GPS lock - Wait 30-60 seconds for satellite acquisition

Missing OBD parameters? - Your vehicle may not support all parameters - Only ECU-supported parameters are displayed - Older vehicles support fewer parameters

Dashboard widgets not updating? - Ensure OBD adapter is connected (check Connect screen) - Verify the metric is supported by your vehicle - Check Details screen to see raw values - For custom PIDs, verify ECU header and PID configuration

Custom PID shows “NODATA”? - Try different ECU headers (7E0, 7E1, 760, etc.) - Verify mode and PID values with terminal app - Some PIDs only work when vehicle is moving - Check if your vehicle supports the specific ECU

Battery draining fast? - Disable accelerometer if not needed (Settings) - Reduce screen brightness - Close other GPS apps running in background - Limit custom PID polling frequency

Map not showing track? - Ensure trip had GPS data during recording - Check if GPS was enabled during trip - Verify trip file contains valid coordinates - Try reopening Map View from Trip Summary

Map tiles not loading? - Check internet connection (required for OpenStreetMap tiles) - Wait for tiles to load on slow connections - Try panning/zooming to refresh tile cache

## 📁 Trip Logs

Format: JSON files with complete trip data
Filename: <ProfileName>_obdlog_<YYYY-MM-DD_HHmmss>.json
Location: Chosen log folder (or Downloads by default)
Contains: All OBD-II readings, GPS coordinates, accelerometer data, trip statistics, custom PID values

Viewing logs: - Use any JSON viewer or text editor - Import into spreadsheet software for analysis - Share via email, cloud storage, or USB

## 🔒 Permissions Required

| Permission | Why it’s needed |
| --- | --- |
| Bluetooth | Connect to OBD-II adapter |
| Location | GPS tracking for distance/speed + required for Bluetooth scanning on Android 12+ |
| Foreground Service | Keep trip recording active in background |
| Internet | Load OpenStreetMap tiles for Map View |

Privacy: All data stays on your device. No data is sent to external servers.

## 📱 System Requirements

- Android 8.0 (API 26) or higher - Supports Android 8.0 through Android 15

- Bluetooth Classic support (all modern phones)

- GPS for trip tracking

- Accelerometer (optional, for road quality analysis)

Compatible Adapters: - Any ELM327 Bluetooth adapter (Classic BT, not BLE) - Recommended: BAFX, OBDLink, Veepeak brands

Android Version Support: - ✅ Android 8.0 (Oreo) - API 26 - ✅ Android 9.0 (Pie) - API 28
- ✅ Android 10.0 (Q) - API 29 - ✅ Android 11.0 (R) - API 30 - ✅ Android 12.0 (S) - API 31 - ✅ Android 13.0 (Tiramisu) - API 33 - ✅ Android 14.0 (Upside Down Cake) - API 34 - ✅ Android 15.0 (Vanilla Ice Cream) - API 36

Note: Android 12+ requires additional Bluetooth permissions that are automatically handled by the app.

## 🆘 Support & Resources

App Version: Check Settings screen for current build number
Documentation: See README.md and ARCHITECTURE.md in app repository
Versioning: Build numbers auto-increment — use for bug reports

Common Questions:

Q: Can I use this with multiple vehicles?
A: Yes! Create a separate profile for each vehicle and switch between them.

Q: Does this work with diesel engines?
A: Yes, select “Diesel” as fuel type in your vehicle profile.

Q: Will this drain my car battery?
A: No. The OBD-II port is powered only when ignition is on. Always unplug the adapter when not in use for extended periods.

Q: Can I export trip data?
A: Yes, enable logging in Settings. Trip logs are saved as JSON files you can share or analyze.

Q: Why are some metrics showing “—” or blank?
A: Your vehicle’s ECU doesn’t support those parameters. Only supported sensors are polled.

Q: Can I add custom parameters for my specific vehicle?
A: Yes! Use the Custom PID feature to add manufacturer-specific parameters. See Custom PID Guide for details.

Q: How do I find custom PIDs for my vehicle?
A: Use the built-in PID discovery tool or a terminal app to explore your vehicle’s supported PIDs.

Q: Can I use custom PIDs from multiple ECUs?
A: Yes, you can configure PIDs from different ECUs (engine, transmission, ABS, etc.) using appropriate headers.

Q: Will custom PIDs affect app performance?
A: Custom PIDs are polled at a slower rate to minimize impact on performance.

Q: Can I view my trips on a map?
A: Yes! After recording a trip, open Trip Summary and tap the map icon to view your route on OpenStreetMap.

Q: Why is my trip track not showing on the map?
A: Ensure GPS was enabled during the trip and that the trip contains valid GPS coordinates.

Q: Do I need internet for Map View?
A: Yes, Map View requires an internet connection to load OpenStreetMap tiles. The GPS track data itself is stored locally.

Q: Can I export trip data from Map View?
A: You can copy individual sample JSON data using the Details button. For full trip export, enable logging in Settings.

## 🎯 Quick Reference Card

| Action | How to do it |
| --- | --- |
| Connect to car | Connect tab → Tap device name |
| Start trip | Trip tab → Start Trip button |
| Pause trip | Trip tab → Pause button |
| Stop trip | Trip tab → Stop Trip button |
| View trip on map | Trip Summary → Tap map icon |
| Navigate map | Drag scrub bar to move cursor |
| View sample data | Map View → Details button |
| View live data | Details tab (full table) or Dashboards tab (gauges) |
| Create dashboard | Dashboards → ⋮ menu → New Layout |
| Add vehicle | Settings → + Add Profile |
| Switch vehicle | Settings → Tap profile name |
| Enable logging | Settings → Enable Logging toggle |
| Choose log folder | Settings → Change Log Folder |
| Manage custom PIDs | Settings → Edit profile → Manage Custom PIDs |
| Discover PIDs | Custom PID list → Discovery button |
| Add custom PID | Custom PID list → + Add button |

Drive safe and enjoy your OBD2 Viewer experience! 🚗💨